A Course In Wood Turning

PREFACE

This book is the outgrowth of problems given to high school pupils by the writers,

and has been compiled in logical sequence. Stress is laid upon the proper use of

tools, and the problems are presented in such a way that each exercise, or project,

depends somewhat on the one preceding. It is not the idea of the writers that all

problems shown should be made, but that the instructor select only such as will

give the pupils enough preliminary work in the use of the tools to prepare them for

other models following.

The related matter on the care of the lathe and tools, the grinding of chisels, the

polishing of projects, and the specific directions and cautions for working out the

various exercises and projects with the drawings, make the book not only valuable

for reference, but also as a class text to be studied in connection with the making of

projects. The drawings show exact dimensions and are tabulated in the upper right-

hand corner in such a way that they may be used in a filing case if desired. At least

two designs are shown for each model, and these may be used as suggestions from

which students, with the aid of the instructor, may work out their own designs.

The book has been divided into two parts: (A) Spindle Turning, and (B) Face-Plate

Turning.  The  same  order  is  followed  in  each  part;  the  related  information  is

supplied where required as the pupil progresses.

Part A takes up the following: (I) Exercises; (II) Models, involving the same tool

processes, only in a somewhat different degree; (III) Oval Turning, explaining the

use of two centers; (IV) Duplicate Turning, where identical pieces are turned.

Part B is arranged as follows: (I) Exercises; (II) Models, which are an application

of cuts in exercises that involve only face-plate work; (III) Models, which require

chucking; (IV) Assembling Exercises, involving spindle turning, face-plate work

and chucking; (V) Spiral Turning, showing the method of turning a spiral on the

lathe.

The ultimate aim of this book is to give, through the exercises and problems, a

thorough understanding of the principles of wood turning by gradually developing

the confidence of the pupil in the complete control of his tools, at the same time

suggesting harmonious lines in design which will lead to other ideas in designing

problems.

TABLE OF CONTENTS

Pages

CHAPTER I

9-10

Introductory

--Commercial and Educational Values of Wood Turning

--Elements of Success

CHAPTER II

11-14

The Lathe

--Care of the Lathe

--Speed of the Lathe

--Method of Figuring the Diameter of Pulleys

--Rules for Finding the Speeds and Sizes of Pulleys

--Points on Setting Up the Lathe and Shafting

CHAPTER III

15-18

Wood Turning Tools

--Grinding and Whetting Turning Tools

--The Gouge

--The Parting Tool

--Scraping Tools

CHAPTER IV

19-21

Spindle Turning

--Centering Stock

--Clamping Stock in the Lathe

--Adjusting the Tool Rest

--Position of the Operator at the Lathe

--Holding the Tools

--Use of the Tools in Spindle Turning

22-32

CHAPTER V

Tool Processes in Spindle Turning

--The Roughing Cut

--The Sizing Cut

--The Smoothing Cut

--Testing for Smoothness

--Measuring for Length

--Squaring Ends

--Cutting Off

--Shoulder Cuts

--Taper Cuts

--V Cuts-Concave Cuts

--Convex Cuts

--Combination Cuts

--Chisel Handles

--Mallets and Handles

--Vise Handles

CHAPTER VI

33-34

Oval Turning

--Tool Operations

CHAPTER VII

35

Duplicate Turning

--Use of Measuring Stick

--Use of Templets

CHAPTER VIII

36-38

Finishing and Polishing

--Ordinary Cabinet Finishing

--French Polishing

--Method of Applying French Polish

CHAPTER IX

39-40

Face-Plate and Chuck Turning

--Methods of Fastening Stock

--Small Single Screw Face-Plate

--Large Surface Screw Face-Plate

--Gluing to Waste Stock

--Lathe Adjustments

--Position of Tool Rest

41-48

CHAPTER X

Tool Processes in Face-Plate and Chuck Turning

--Straight Cuts

--Roughing Off Corners

--Calipering for Diameter

--Smoothing Cut

--Roughing Cut on the Face

--Smoothing the Face

--Laying Off Measurements

--External Shoulders

--Internal Shoulders

--Taper Cuts

--V Cuts

--Concave Cuts

--Convex Cuts

--Combination Cuts

--Use of Scraping Tools

--Internal Boring

--Turning a Sphere

CHAPTER XI

49-55

Spiral Turning

--Single Spiral, Straight Shaft

--Tapered Shaft

--Double Spiral, Tapered Shaft

--Double Spiral, Straight Shaft

--Double Groove Spiral, Straight Shaft

PLATES--SPINDLE TURNING.

Straight Cuts

57

Shoulder Cuts

59-65

Taper Cuts

67-77

V Cuts

79-81

Concave Cuts

83-87

Convex Cuts

89-95

Combination Cuts

97-101

Chisel Handles

103-107

Cabinet File Handle

109

Scratch Awl Handle

111

Carving Tool Handle

113

Turning Chisel Handle

115

Mallets

117-119

Gavels

121-127

Darning Eggs

129-133

Stocking Darner

131

Potato Masher

135

Rolling Pins

139-141

Vise Handle

143

Screw Driver Handles

145-147

Pene Hammer Handle

149

Claw Hammer Handle

151

Indian Clubs

153-155

Dumb Bells

157-159

Ten Pins

161

Drawer Pulls

163-165

PLATES--CHUCK TURNING.

Straight Cuts

167-169

Shoulder Cuts

171-173

Taper Cuts

175-177

V Cuts

179-181

Concave Cuts

183-185

Convex Cuts

187-189

Combination Cuts

191-195

Match Boxes

197-201

Pin Trays

203-205

Hair Pin Receivers

207-209

Hat Pin Receivers

211-213

Ornamental Vases

215-219

Spinnet

221

Towel Rings

223-227

Card Trays

229-235

Picture Frames

237-243

Nut Bowls

245-251

Napkin Rings

253-257

Jewel Boxes

259-273

Collar Boxes

275-279

Sphere

281

Checker Men

283

Candle Sticks

285-293

Shaving Stands

295-301

Reading Lamp Stands

303-307

Pedestal

309

Smokers' Stands

311-313

Pin Cushion and Spoon Holder

315

Chess Men

317-319

Pedestals

321-325

Electric Reading Lamps

327-335

Magazine Holders

337-339

CLASSIFICATION OF PLATES

A. SPINDLE TURNING

I. Exercises

1. Straight Cuts,

a

2. Shoulder Cuts,

a

-

b

-

c

-

d

3. Taper Cuts,

a

-

b

-

c

-

d

-

e

-

f

4. V Cuts,

a

-

b

5. Concave Cuts,

a

-

b

-

c

6. Convex Cuts,

a

-

b

-

c

-

d

7. Combination Cuts,

a

-

b

-

c

II. Models

1. Chisel Handles,

a

-

b

-

c

Cabinet File Handle,

d

Scratch Awl Handle,

e

Carving Tool Handle,

f

Turning Chisel Handle,

g

2. Mallets,

a

-

b

3. Gavels,

a

-

b

-

c

-

d

4. Stocking Darners,

a

-

b

Darning Egg,

c

5. Potato Mashers,

a

-

b

6. Rolling Pins,

a

-

b

7. Vise Handles,

a

III. Oval Turning

1. Screw-driver Handles,

a

-

b

2. Hammer Handles

Penne Hammer Handle,

a

Claw Hammer Handle,

b

IV. Duplicate Turning

1. Indian Clubs,

a

-

b

2. Dumb-bells,

a

-

b

3. Tenpins,

a

4. Drawer Pulls,

a

-

b

B. FACE-PLATE AND CHUCK TURNING

I. Exercises

1. Straight Cuts,

a

-

b

2. Shoulder Cuts,

a

-

b

3. Taper Cuts,

a

-

b

4. V Cuts,

a

-

b

5. Concave Cuts,

a

-

b

6. Convex Cuts,

a

-

b

7. Combination Cuts,

a

-

b

-

c

II. Face-Plate Models

1. Match Boxes,

a

-

b

-

c

2. Pin Trays,

a

-

b

3. Hair Pin Receivers,

a

-

b

4. Hat Pin Receivers,

a

-

b

5. Ornamental Vases,

a

-

b

-

c

6. Spinnet, (game)

a

III. Chuck Models

1. Towel Rings,

a

-

b

-

c

2. Card Trays,

a

-

b

-

c

-

d

3. Picture Frames,

a

-

b

-

c

-

d

4. Nut Bowels,

a

-

b

-

c

-

d

5. Napkin Rings,

a

-

b

-

c

6. Jewel Boxes,

a

-

b

-

c

-

d

-

e

-

f

-

g

-

h

7. Collar Boxes,

a

-

b

-

c

8. Spheres,

a

9. Checker Men,

a

IV. Assembling Exercises

1. Candle Sticks,

a

-

b

-

c

-

d

-

e

2. Shaving Stands,

a

-

a’

-

b

-

b’

3. Reading Lamp Stands,

a

-

b

-

c

4. Pedestals,

a

5. Smoking Stands,

a

-

b

6. Pin Cushions and Spool Holder,

a

7. Chess Men,

a

-

a’

V. Spiral Turning

1. Pedestal, (Single)

a

-

a’

, (Double)

b

2. Reading Lamps, (Single)

a

-

a’

-

a’’

(Double)

b

-

b’

3. Magazine Holder,

a

-

a’

CHAPTER I

INTRODUCTORY

Wood turning has had a definite place in the commercial world for a great many

years. It is used in various forms in making furniture and furniture parts, building

trim, tool parts, toys, athletic paraphernalia and many other useful and beautiful

articles in common use.

When  properly  taught  in  the  schools  it  is  one  of  the  most  valuable  types  of

instruction. It appeals to pupils more than any other type of manual work, as it

embodies both the play and work elements. It is very interesting and fascinating

and, in the hands of a skilled instructor, is readily correlated with other work.

Wood turning gives a pupil preliminary experience necessary in pattern making and

machine shop work. It brings into play the scientific element by demonstrating the

laws  governing  revolving  bodies.  In  bringing  the  chisel  into  contact  with  the

revolving  surface,  the  mathematical  principle  of  the  "point  of  tangency"  is

illustrated.  Excellent  tool  technique  is  developed  in  wood  turning  as  on  the

exactness of every movement depends the success of the operator, and any slight

variation will spoil a piece of work. This brings in a very close correlation of the

mental and motor activities and also gives the student an opportunity for observing

and thinking while at work. When his tool makes a "run" he must determine the

reason and figure out why a certain result is obtained when the chisel is held in a

given position. Certain cuts must be fully mastered, and it takes a good deal of

experience and absolute confidence in one's self in manipulating the tools before it

is possible to attempt skilful work. If scraping is allowed the educational value of

the work is lost.

In  wood  turning  a  vast  field  for  design  and  modeling  is  opened,  and  art  and

architecture can be correlated. The pupil will see for himself the need of variety in

curves and must use his judgment in determining curves that are so harmonious and

pleasing that they will blend together. If properly taught the beauty in the orders of

architecture can be brought out in the making of the bead, fillet, scotia, cove, etc.

A  feeling  of  importance  is  excited  in  a  boy  when  he  sees  his  hands  shaping

materials into objects of pleasing form. Wood turning properly taught awakens the

aesthetic sense and creates a desire for the beautiful. The boy or man who has

learned  to  make  graceful  curves  and  clean-cut  fillets  and  beads  will  never  be

satisfied with clumsy effects which are characteristic in cheap commercial work,

made only to sell.

Success in turning depends on the following:

1. Care of lathe, tools, selection of materials.

2. Study of the scientific elements of--

a. Revolving bodies.

b. Points of tangency.

c. Study of results by reasoning and observing.

3. Development of technique and exactness.

4. Correlation of mental and motor activities.

CHAPTER II

THE LATHE

The sizes of turning lathes are given as 10", 12", etc. These figures denote the

diameter, or size, of the largest piece of work that can be turned on them. The

measurement is taken from the center point of the live center to the bed of the lathe

(usually 5" or 6") and is one-half the diameter of the entire circle. The length of a

lathe  is determined  by the  length  of  a  piece  of  work  that  can  be turned.  This

measurement is taken from the points of the live and dead centers when the tail

stock is drawn back the full extent of the lathe bed. Fig. 1 shows a turning lathe

with sixteen principal parts named. The student should learn the names of these

parts and familiarize himself with the particular function of each.

CARE OF THE LATHE

The lathe should be oiled every day before starting. At the end of the period the

lathe should be brushed clean of all chips and shavings, after which it should be

rubbed off with a piece of waste or cloth to remove all surplus oil. All tools should

be wiped clean and put in their proper places. If a student finds that his lathe is not

running as it should, he should first call the attention of the instructor to that fact

before attempting to adjust it; and then only such adjustments should be made as

the instructor directs.

SPEED OF THE LATHE

The speed of the lathe should range from 2400 to 3000 revolutions per minute

when the belt is on the smallest step of the cone pulley. At this speed stock up to 3"

in diameter can be turned with safety. Stock from 3" to 6" in diameter should be

turned on the second or third step, and all stock over 6" on the last step. The speed

at which a lathe should run depends entirely upon the nature of the work to be done

and the kind of material used. Pieces that cannot be centered accurately and all

glued-up work with rough corners should be run slowly until all corners are taken

off and the stock runs true. At high speed the centrificial force on such pieces is

very great, causing the lathe to vibrate, and there is a possibility of the piece being

thrown from the lathe thus endangering the worker as well as those around him.

After the stock is running true the speed may be increased.

Fig. 1. - Wood Turning Lathe

TO FIGURE THE DIAMETER OF PULLEYS

Suppose a motor runs 1500 R.P.M. and is fitted with a 4" pulley. Suppose also, a

main shaft should run 300 R.P.M.

Then, 1500 : 300 :: x : 4;

Or, 300x = 6000,

x = 20, or the diameter of the large pulley on the main shaft.

Suppose again that a line shaft runs 300 R.P.M., and a counter shaft 600 R.P.M.

The counter shaft has a pulley 4" in diameter. The pulley on the line shaft must

then have a diameter of 8".

300 : 600 :: 4 : x;

Or, 300x = 2400,

x = 8"

Suppose the cone pulley on the counter shaft runs 600 R.P.M.; a lathe spindle runs

2200 R.P.M., when connected with the small cone pulley which has a diameter of

3". The large cone pulley has then a diameter of 11".

600 : 2200 :: 3 : x

Or, 600x = 6600;

x = 11"

RULES FOR FINDING THE SPEEDS AND SIZES OF PULLEYS

1. To find the diameter of the driving pulley:

Multiply the diameter of the driven by the number of revolutions it should make

and divide the product by the number of revolutions of the driver. (20 x 300 =

6000; 6000 ÷ 1500 = 4"--diameter of motor pulley.)

2. To find the diameter of the driven pulley:

Multiply the diameter of the driver by its number of revolutions and divide the

product by the number of revolutions of the driven. (4 x 1500 = 6000; 6000 ÷ 300 =

20"--diameter of the driven pulley.)

3. To find the number of revolutions of the driven pulley:

Multiply the diameter of the driver by its number of revolutions and divide by the

diameter of the driven. (4 x 1500 = 6000; 6000 ÷ 20 = 300--revolutions of driven

pulley.)

POINTS ON SETTING UP LATHE AND SHAFTING

The counter shaft should be about 7' above the lathe. A distance of 6' from the

center of the shaft to the center of the spindle is sufficient. In setting a lathe or

hanging a counter shaft it is necessary that both be level. The counter shaft must be

parallel to the line shaft. When the counter shaft is in position a plumb bob should

be  hung  from  the  counter  shaft  cone  to  the  spindle  cone;  the  lathe  should  be

adjusted so that the belt will track between the two cone pulleys. The axis of the

lathe must be parallel to that of the counter shaft. The lathe, however, need not be

directly  beneath  the  counter  shaft  as  the  belt  will  run  on  an  angle  as  well  as

perpendicular.

CHAPTER III

WOOD TURNING TOOLS

A wood turning kit should consist of one each of the following tools. Fig. 2 shows

the general shape of these tools.

1¼" Gouge

¾" Gouge

½" Gouge

¼" Gouge

1¼" Skew

¾" Skew

½" Skew

¼" Skew

?" Parting Tool

½" Round Nose

¼" Round Nose

½" Square Nose

¼" Square Nose

½" Spear Point

½" Right Skew

½" Left Skew

Slip Stone with round edges

6" Outside Calipers

6" Inside Calipers

8" Dividers

12" Rule

½ pt. Oil Can

Bench Brush

GRINDING AND WHETTING TURNING TOOLS

Skew Chisel

The skew chisel is sharpened equally on both sides On this tool the cutting edge

should form an angle of about 20° with one of the edges. The skew is used in

cutting both to the right and to the left, and therefore, must be beveled on both

sides. The length of the bevel should equal about twice the thickness of the chisel at

the point where it is sharpened. In grinding the bevel, the chisel must be held so

that the cutting edge will be parallel to the axis of the emery wheel. The wheel

should be about 6" in diameter as this will leave the bevel slightly hollow ground.

Cool the chisel in water occasionally when using a dry emery. Otherwise the wheel

will burn the chisel, taking out the temper; the metal will be soft and the edge will

not stand up. Care should be exercised that the same bevel is kept so that it will be

uniformly  hollow  ground.  The  rough  edge  left  by  the  emery  wheel  should  be

whetted off with a slip stone by holding the chisel on the flat side of the stone so

that the toe and heel of the bevel are equally in contact with it. Rub first on one side

and then on the other. The wire edge is thus worn off quickly as there is no metal to

be worn away in the middle of the bevels. The chisel is sharp when the edge, which

may be tested by drawing it over the thumb nail, is smooth and will take hold

evenly along its entire length. If any wire edge remains it should be whetted again.

Fig. 2. - Lathe Tools

Gouge

The gouge used in wood turning is beveled on the outside and is ground so that the

nose  is  approximately  semi-circular in shape.  The  tool is a  combination of the

round nose chisel and the ordinary gouge. The bevel should extend well around to

the ends so that the cutting edge extends to each side. This is necessary to avoid the

abrupt corners which would be present if the nose were left straight across as in the

ordinary wood-working gouge. In making shearing cuts the round nose permits the

tool to be rolled to the side to avoid scraping the work. The length of the bevel

should be about twice the thickness of the blade at the point where the sharpening

begins.

The sharpening of a gouge for turning is rather difficult for the average student.

The ordinary gouge which has a square nose may be beveled by merely turning it

half way around and back again. In working out the round nose of a gouge for

wood turning, it is necessary that the handle be swung from one side to the other

while,  at  the  same  time,  the  chisel  is  revolved  to  cut  the  bevel  evenly.  It  is

sometimes necessary to allow some pupils to use the side of the emery wheel in

sharpening  the  gouge.  This  kind of  grinding, however,  does  not leave the  tool

hollow ground as when the face of the wheel is used.

To complete the sharpening the rough edge is worked smooth on a slip stone, the

cross section of which is wedge-shaped and the edges of which are rounded. The

toe and heel of the beveled side of the gouge are brought into contact with the flat

side of the stone. As the sharpening proceeds the wire edge is worked to the inside

of the gouge. The rounded edge of the stone is then placed inside the gouge and is

worked back and forth until the rough edge disappears. Great care must be taken

not to bevel the inside of the gouge when whetting with the round edges of the

stone, as the result will be the same as with an ordinary chisel or plane bit.

Parting Tool

The parting tool is sharpened on both sides. This tool differs from the ordinary

chisel in that it is between ?" and ¾" thick and only about ?" wide at the widest

point, which is in the center of its entire length. The bevels must meet exactly at the

center, or the widest point, and should make an angle of about 50° with each other.

If the bevels do not meet at the widest point the tool will not clear, and the sides

will rub against the revolving stock; the tool will be burned and will thus lose its

temper. The bevel should be hollow ground slightly as then comparatively little

metal need be removed when whetting.

Scraping Tools

The round nose, square nose, spear point, right skew and left skew are scraping

tools, used chiefly in pattern work and sometimes in face-plate work. They are

sharpened on one side only, and the bevel is about twice the thickness of the chisel

at the point  where  sharpened.  These  tools  should be slightly hollow  ground to

facilitate the whetting. Scraping tools become dull quite easily as their edges are in

contact with the wood almost at right angles. After sharpening, the edges of these

tools may be turned with a burnisher or the broad side of a skew chisel in the same

manner that the edge of a cabinet scraper is turned though not nearly to so great a

degree. This will help to keep the tool sharp for, as the edge wears off, the tool

sharpens itself to a certain extent. The chisel is of harder material than a cabinet

scraper so that it will not stand a great amount of turning over on the edge. Small

pieces will be broken out, unless a flat surface is rubbed against the edge at a more

acute angle than was used in the whetting. If a narrow burnisher is used, pieces are

more likely to be broken out from the sharp edge and thus make the tool useless.

CHAPTER IV

SPINDLE TURNING

Spindle turning is the term applied to all work done on a lathe in which the stock to

be worked upon is held firmly between the live and dead centers. There are two

methods in common use  in wood turning: first, the scraping or pattern-makers'

method;

and

second,

the

cutting

method.

Each

has

its

advantages

and

disadvantages, but it is necessary that both be learned in order to develop a well

rounded turner. Care should be exercised, however, that each method be used in its

proper place. The first is slower, harder on the cutting edge of tools, and less skill is

required to obtain accurate work; the second is faster, easier on the cutting edge of

tools, and the accuracy of results obtained depends upon the skill acquired. As skill

is the one thing most sought for in high school work, the use of the cutting method

is advocated entirely for all spindle turning and, with but few exceptions, for face-

plate and chuck turning.

TO CENTER STOCK

If the wood to be turned is square or rectangular in shape the best way to locate the

center is to draw diagonals across the end of the stock. The point of intersection

locates the center.

CLAMPING STOCK IN THE LATHE

Take the live center from the spindle and with a wooden mallet drive the spur deep

into  the  wood.  Never drive the wood  onto  the live center  while  in the spindle

because serious injury may be done the machine by such practice. When extremely

hard wood is being used, it is a good practice to make saw cuts along the diagonal

lines and bore a hole at the intersection, thus allowing the spur to enter the wood

more freely. Oil the other end of the wood while holding it in a vertical position,

and give the oil a chance to penetrate into the wood. Then replace the live center by

taking the stock and center and forcing it into the spindle by a sudden push of the

hand. The tail stock is then moved about ½" to 1" from the end of the piece to be

turned, having the tail spindle well back in the tail stock. The tail stock is then

clamped to the lathe bed. Turn the tail stock hand wheel until the wood is held

firmly. Work the cone pulley by hand at the same time, so that the cup or dead

center will be forced deeply into the wood, so deeply that the live center will not

continue to turn. Now turn the dead spindle back until the live spindle begins to

turn freely and clamp the dead spindle fast.

Fig. 3

ADJUSTING THE TOOL REST

Horizontally the tool rest should be set about ?" from the farthest projecting corner

of the wood and should be readjusted occasionally as the stock diminishes in size.

The vertical height varies slightly according to the height of the operator. It is even

with the center of the spindle for a short person; ?" above for a medium person;

and ¼" above for a tall person. So long as the stock is in its square form the tool

rest should never be adjusted while the machine is in motion as there is danger of

the rest catching the corners and throwing the stock from the machine. Also see that

everything is clamped tight before starting the lathe.

POSITION OF THE OPERATOR

The operator stands firmly on the floor back far enough from the lathe to allow him

to pass the tools from right to left in front of his body without changing the position

of the feet. It may be found convenient to turn slightly, bringing the left side of the

body a little closer to the lathe. In no case, however, should the tools be brought in

contact  with  the  body  as  the  cutting  operation  from  right  to  left  should  be

accomplished by a movement of the arms alone and not the swaying of the body.

(Fig. 3.)

HOLDING THE TOOLS

All tools should be held firmly but not rigidly. The right hand should grasp the

handle  at  the  extreme  end  for  two  reasons:  first,  to  give  as  much  leverage  as

possible so that the tool will not be thrown from the hands in case it should catch in

the wood; second, a slight wavering of the hand will not cause as much variance in

the cuts as when held closer up to the rest. The left hand should act as a guide and

should be held over the tool near the cutting edge. The little finger and the back

part  of the palm  of  the  hand should touch the tool rest  thus  assuring a steady

movement. The left hand should not grasp the tool at any time. (Fig. 3.)

USE OF THE TOOLS IN SPINDLE TURNING

The correct use of the various tools used in spindle turning will be explained in

detail as the steps are worked out in the sequence of operations on the exercises in

Section A-I.

CHAPTER V

TOOL PROCESSES IN SPINDLE TURNING

Exercise

A-I--1-a

. Straight Cuts

1. THE ROUGHING CUT (LARGE GOUGE).

FIG. 4. Place the gouge on the rest so that the level is above the wood and the

cutting edge is tangent to the circle or surface of the cylinder. The handle should be

held well down.

Fig. 4.

Roll the gouge over slightly to the right so that it will make a shearing cut instead

of  a  scraping  cut.  This  rolling  of  the  tool  will  also  throw  the  chips  from  the

operator.

Then lift the handle slowly, forcing the cutting edge deep enough into the wood to

remove all or nearly all of the corners, at the end of the work which is being turned.

This cut is begun  about  ¾"  from the dead center end. Work back another ¾",

moving toward the live center and make a second cut, and so on until the entire

length of the cylinder is gone over. This method of removing corners should always

be followed to avoid any possibility of breaking a large sliver from the stock, with

consequent danger to the worker.

The tool may then be worked from one end to the other, getting a fairly-smooth,

regular surface, slightly above the diameter required. However, do not begin on the

very edge of the cylinder end. It is better to begin about 2" from one end and work

to the other, and then reverse and work back.

The tool should also be held at a slight angle to the axis of the cylinder, with the

cutting point always in advance of the handle.

Fig. 5.

2. THE SIZING CUT (SMALL GOUGE). FIG. 5.

Set the calipers to the required diameter of the cylinder.

With a small gouge held in the right hand scrape grooves about 1" apart, holding

the calipers in the left hand perpendicular to the cylinder and measuring the cuts as

they are made. The scraping should continue until the calipers will pass easily over

the cylinder. It will be well while scraping to work the handle of the gouge a little

from side to side so that the nose has more clearance. This will prevent the piece

which is being turned from chattering or vibrating.

The calipers will be slightly sprung by coming in contact with the revolving stock

but this error in diameter will be removed by the finishing cut which removes these

marks from the finished cylinder.

3. THE SMOOTHING CUT (LARGE SKEW).

FIG. 6. Lay the skew chisel on the rest with the cutting edge above the cylinder and

at an angle of about 60° to the surface.

Slowly draw the chisel back and at the same time raise the handle until the chisel

begins to cut about ¼" to ?" from the heel. The first cut is begun from 1" to 2"

from either end and is pushed toward the near end. Then begin at the first starting

point and cut toward the other end. One should never start at the end to make a cut

as there is danger that the chisel will catch and cause the wood to split or that the

chisel will be torn from the hands.

The first cut takes off the bumps and rings left by the gouge, and takes the stock

down so one can just see where the scraping to size was done. Then take the last

cut and remove all traces of these, leaving the cylinder perfectly smooth and of the

required diameter at each end. Test the cylinder for accuracy with a straight edge.

Fig. 6.

4. TESTING FOR SMOOTHNESS. In testing for smoothness place the palm of the

hand, with the fingers extended straight, lightly on the back of the cylinder opposite

the tool rest. This position will avoid any possibility of the hand being drawn in

between the cylinder and the rest.

Fig. 7.

5. MEASURING FOR LENGTH (RULE AND PENCIL). FIG. 7. Hold the back

edge of the rule in the left hand and place it on the tool rest so that the front edge is

almost in contact with the revolving cylinder.

With a sharp pencil mark off the required length, starting from the dead center end.

The first mark should be just far enough in on the cylinder to insure cutting past the

point of the dead center. This will leave all surplus stock at the live center end

where it is needed, because, if not enough stock is left at this end, there is danger of

striking the live center spur with the tool and of injuring the chisel and perhaps the

work.

In case several measurements are to be made, as in some of the following exercises,

the rule should not be moved until all are marked. This will insure more accurate

work than if the rule be changed several times.

6. SQUARING ENDS (SMALL SKEW AND PARTING TOOL). FIG. 8. This

operation is done with the toe or acute angle of the ½" or ¼" skew chisel.

Place the chisel square on the tool rest. Swing the handle out from the cylinder so

that the grind, which forms the cutting edge, next to the stock is perpendicular to

the axis of the cylinder. The heel of the chisel is then tipped slightly from the

cylinder in order to give clearness. Raise the handle and push the toe of the chisel

into the stock about ?" outside the line indicating the end of the cylinder. Swing

the handle still farther from the cylinder and cut a half V. This will give clearance

for the chisel point and will prevent burning. Continue this operation on both ends

until the cylinder is cut to about 3/16" in diameter.

The remaining ?" is then removed by taking very thin cuts (about 1/32") holding

the  chisel  as  first  stated.  After  each  cut  is  made  the  end  should  be  tested  for

squareness by holding the edge of the chisel over the end of the cylinder.

Fig. 8.

This is an easy cut after it is mastered, but is one of the hardest to learn. Should the

operator lose control of the tool and allow any part other than the point to touch the

cylinder, a run or gashing of the wood will be caused.

In large cylinders where considerable stock has to be cut away in order to square

the ends, time will be saved by sizing the ends down with the parting tool to within

?" of the desired line, leaving enough stock at the base of the cuts to still hold the

cylinder rigid while cutting on the ends.

Fig. 9.

For this operation hold the parting tool on the rest with the cutting edge parallel to

the axis of the cylinder and the lower grind tangent to the cylinder. Lift the handle

and force the cutting edge into the wood; at the same time push the chisel forward

to keep it at the proper tangency.

7. CUTTING OFF (SMALL SKEW). FIG. 9.

After  both  ends  have been squared cut away stock,  at both ends, to leave just

enough to hold the cylinder from separating from the waste ends.

With the chisel held in the right hand in the same position as in squaring the ends,

and the fingers of the left hand around the stock to catch it, slowly force the point

of the chisel into the stock at the live center end, until it is cut free and the cylinder

stops in the operator's hand. Too much pressure should not be used in this operation

or it will cause the cylinder to twist off instead of being cut, and will leave a ragged

hole in the end.

The dead center end, which has been scored heavily before cutting off at the live

center, is then removed by holding the grind of the chisel flat on the end of the

cylinder. The latter is revolved by hand until the stock is cut away.

Exercise

A-I--2-a

. Shoulder Cuts

1. Turn a cylinder to the largest diameter required.

2. Lay off measurements with rule and pencil.

3. With the gouge (where space permits) or the parting tool (in narrow spaces)

rough out surplus stock, keeping 1/16" away from the lines indicating shoulders.

4. Caliper to the diameter of the second step.

5. The shoulders are cut down as described in "Squaring Ends, Step 6, Straight

Cuts."

6. The new diameter or step is then trued up with a skew chisel in the same manner

as a cylinder; except that in nearing the shoulder the chisel is pushed up on the

cylinder until the heel, which is the only part that can be worked into the corner,

becomes the cutting point. Fig. 10. In very narrow steps it will be advisable to use

the heel entirely as a cutting point.

In spaces between shoulders, too narrow to permit the use of the skew chisel, very

effective work can be accomplished by slightly tipping the parting tool sideways to

allow a shearing cut to be taken with the cutting edge.

7. Where several steps are required on the same cylinder, each successive one is

worked out as above described.

Note:--All preliminary steps in working stock to size, laying of dimensions, etc., in

preparation for the exercise in hand, will be omitted in the following exercises:

Exercise

A-I--3-a

. Taper Cuts

Fig. 10.

Calipering for New Diameters. For all diameters on tapers the calipers should be

set 1/16" larger than the desired measurement in order to avoid working under size

in the finishing cut which removes all caliper marks.

If the taper runs to the extreme end of the cylinder, as in Plate

A-I--3-a

, a parting

tool should be used, instead of a gouge, to take off a very thin shoulder.

If the taper forms an internal angle as in Plate

A-I--3-b

, a gouge is used as in Step

2--Sizing Cut--Plate

A-I--1-a

.

In other cases where tapers connect with straight cylindrical shoulders it is best to

turn the shoulders to size before working the tapers.

In cutting a long straight taper the skew chisel is used, much in the same manner as

in ordinary cylinder work, except that at the start of each cut the heel must be the

cutting point. This will avoid any chance of the chisel catching and drawing back

and thus gouging the wood beyond the starting point. As soon as the cut is well

under way the chisel may be pushed up on the cylinder so that the cutting point is a

little above the heel. All cuts should be made from the highest point on the cylinder

to the lowest and thus cut across the grain of the wood.

In making the cut, care should be taken to see that the chisel is not tipped to a

greater angle than that of the taper wanted. Should that be done a hollow, or dished

out, taper is sure to be the result instead of a straight one.

Exercise

A-I--4-a

. V Cutting

In cutting V's a small skew is almost always used and the cutting is done with the

heel.

Place the chisel square on the tool rest so that the cutting edge is perpendicular to

the axis of the cylinder. Draw the chisel back and raise the handle so that the heel is

driven into the wood, thus scoring it. This cut should not be too deep or the chisel

will burn. This scoring should be at the exact center of the V cut.

Swing the handle a little to the right and at the same time tip the chisel so that the

grind, which forms the cutting edge, is at an angle of about 45° with the axis of the

cylinder. The handle is then raised at an angle of 45° bringing the heel down to

make a good cut. The chisel is then swung to the other side and a similar cut is

taken. These cuts are continued, together with the center scoring, until quite close

to the pencil marks. Test the angle before the finishing cut is taken.

It will be found best to have the V slightly greater than 90° at the base until the

final cut is made, at which time it can be trued up.

The V should be tested with the square end of a rule. The cylinder should not be in

motion while testing.

When angles other than 45° are cut, the cutting edge of the chisel should be tipped

so that it is parallel or nearly so to the side of the cut desired.

A-I--5-a

. Concave Cuts

The concave cuts as a rule will give the pupil considerable trouble at first owing to

the  fact  that  the  grind,  which  forms  the  cutting  edge  and  which  must  be held

perpendicular to the cylinder at the start, is on the under side of the tool and cannot

be seen. However, as soon as the correct angle of the tool is located, the cut will be

found as easy as any. Concaves are usually made with a medium sized gouge either

the ½" or ¾".

Place the gouge on the rest with the grind or cutting edge well above the wood. The

tool is then rolled on its side so that the grind at the cutting point, which is on the

lip of the gouge well below the center, is perpendicular to the axis of the cylinder.

Fig. 11.

Slowly raise the handle to force the gouge into the wood. As soon as the gouge has

taken hold,  the  tool is  forced forward  and  upward by  a  slight  lowering of  the

handle, while at the same time it is rolled back toward its first position. Care should

be taken not to roll the chisel too fast or a perfect arc will not be cut.

Fig. 11.

By this triple action the grind, which comes in contact with the surface of the curve,

forces the lip sidewise and cuts one quarter of a circle. Reverse the position of the

gouge and cut from the other side in the same manner to form the other half of the

semi-circle. The cutting should always stop at the base of the cut as there is danger

that the tool will catch when cutting against the grain of the wood on the other side.

Repeat this operation until within about 1/16" of the required size. At the end of

each  successive  cut  the  tool  should  have  been  forced  far  enough  forward  and

upward to bring the grind or nose of the chisel well out on top of the cut. Fig. 12.

The exact depth of the concave is then calipered in the usual manner as described

before. A finishing cut is then taken after the cut has been tested with a templet.

Fig. 12.

A-I--6-a

. Convex Cuts

The convex cut, or Bead as it is usually called, is generally considered the hardest

cut to make.--The cut is made with the heel of a small skew chisel, usually the ¼"

or ?".

After the cylinder has been marked off, rough out all stock between the beads with

a parting tool. The base of the cuts is finished the same as described in Plate

A-I--1-

a

, for shoulder cutting. With a sharp pencil mark the center of each bead to be

made. This line is the starting point for all cutting.

Place the chisel on the rest, with the cutting edge above the cylinder and the lower

grind tangent to it. Draw the chisel back and raise the handle to bring the heel of

the chisel in contact with the cylinder at the line indicating the center of the bead.

The chisel is then moved to the right (if cutting the right side of the bead); at the

same time the chisel is continually tipped to keep the lower grind tangent to the

revolving cylinder and also to the bead at the point of contact. Fig. 13. This cut is

continued until the bottom of the bead is reached. It is well in turning a series of

beads to work the same side of all before reversing to the other side.

Note:--The same principles employed in this exercise are also used in working out

long convex curves such as are found in chisel handles, mallet handles, etc. The

only exception is that in most cases the point of contact need not be the heel of the

chisel but higher up as in ordinary straight work.

A-I--7-a

--Combination Cuts

These exercises are so designed as to include one or more of each of the foregoing

cuts. The student here is given an opportunity of combining these cuts into one

finished product.

Fig. 13.

An  analysis  of the exercise  chosen  should  be  made  to determine  which  of  the

various cuts should be made first, second, etc., in order to produce the exercise in

the shortest time and with the least amount of tool manipulation.

After  the  student  has  mastered  these  cuts  with  a  certain  degree  of  skill  and

accuracy, he is ready to apply them in working out various models in Section II.

A-II--1-a

. Chisel Handles

At this point it is well to state that the small end of all work should be turned at the

dead center. In the case of chisel handles the socket or ferrule end is at the dead

center where the stock can safely be cut away to permit the fitting of the ferrule or

the socket.

After the stock is turned to a cylinder of the largest dimension desired, the taper, for

the socket chisel, should be turned first and fitted to the chisel in which it is to be

used. Then the rest of the handle is worked out. Ferrules should also be fitted in the

same manner. A drive fit should be used for all ferrules.

A-II--2 and 3. Mallets and Gavels

The biggest source of trouble in these models is getting the handles to fit true. This

is caused by not getting the hole in the head straight.

Turn the head to a cylinder 3/16" larger than the finished dimension. Then bore the

hole perpendicular to the axis as near as possible, either by leaving it between the

lathe centers or by placing it in a vise. The handle is then fitted into the head. A

snug fit is necessary. If one side "hangs" or is lower than the other the centers are

moved sufficiently to correct it. The head is then turned to exact size and finished.

A-II--7. Vise Handles.

Turn the spindle with the solid head to dimensions. Bore a hole through a 1¼"

square block and fit the block snugly to the end of the spindle. Turn this block to

the same dimensions as the other head. This method will save chucking the second

head and is much quicker.

CHAPTER VI

OVAL TURNING

Oval work as a problem in turning will be found to be a very good one as well as

interesting to the pupil. It brings in the principle of the oval as used in ordinary

shop practice; (arcs from points on the major and minor axes). For thick heavy

ovals the off-centering is very slight, while for long, thin ones the off-centering is

greater. The measurements given on Plates

A-III--1-a

, b and

A-III--2-a

, b will give

a good idea of approximate distances to be used.

While the tool operations are much the same as in other spindle turning there is one

notable difference. The design must be worked out by eye, because of the nature of

the work no caliper measurements can be made for depth of cuts.

To get the best results the stock of oval turning should be cut square or slightly

rectangular in cross-section and about 3" longer than the model to be made. The

thickness of the stock should be about ?" greater than the major axis of the oval

wanted.

The centers are located in the usual manner after which perpendicular lines are

drawn from the sides, passing through the points of the centers. From the ends of

one of these, perpendicular lines are extended lengthwise of the stock (on opposite

sides) meeting the corresponding perpendicular at the other end of the stock. These

lines form the ridge of the oval. On the other perpendiculars, the points for off-

centering are laid off, measuring the required distance on both sides of the center

point.

With a ?" drill bore holes ¼" deep at each of the off-centering points as well as the

original center. This will insure the lathe centers penetrating the stock at the proper

point. The stock is then placed in the lathe, using two corresponding off-center

points as centers.

With the lathe running at third speed turn down the stock to the horizontal line

forming the ridge of the oval, excepting for a distance of about 1¼" at the ends.

The stock at the ends is necessary for the off-centering and, if cut away, will spoil

the centering for the other side, especially at the live center end. The stock is then

changed to the other off-center points and the second side is cut down to the line.

All measurements are then laid out and the design is cut, changing the stock in the

lathe when necessary. Care should be taken that the sharp ridge left on the work

forms a straight line the full length of the stock. After the design has been finished,

the stock is centered on the true center and a very thin cut is taken the full length of

the object to remove the sharp corners. The model is then sandpapered while the

lathe is running very slowly.

CHAPTER VII

DUPLICATE TURNING

Under  the  head of  duplicate  turning  have  been  classified only  such  models  as

clearly indicate the necessity of making two or more articles to complete the model

or set of articles desired. But it is not intended to convey the idea that other models

may not  be  made  in  duplicate  as  in  many  cases  it  is  very  desirable and even

preferable that they should be made that way. Whatever the problem may be the

suggestions offered at this point may be applied effectively.

Whenever two or more models are to be made identically alike there are always

two  possibilities  of  inaccuracies  that  will  render  the  work  dissimilar:  First,

inaccurate measuring both for length and points of new diameters and also on the

new diameters themselves; second, a variation in the curved surfaces either on long

convex or concave cuts.

The first difficulty can be overcome to a great extent by the use of a measuring

stick. This stick should be made of any soft wood. It should be straight on one edge

and about the  thickness of  an  ordinary rule. On the straight  edge  lay  off  very

carefully measurements for length, shoulders, beads, concaves and all points where

calipering for new diameters will be necessary. Insert at each point measured a

small brad which has been sharpened at both ends, leaving the end protrude about

?". Care should be taken that all brads protrude the same distance.

After the stock has been turned to the largest diameter, the stick is held in the

position of the rule while measuring and the points are forced against the revolving

cylinder, thus scoring it. This stick can be used as many times as the model is to be

made and the measurements will always be the same.

To avoid dissimilar curves it is well to cut out a full sized templet of the model to

be made. This templet can be made of any thin, stiff material, preferably light sheet

iron. In some cases it will be necessary to make the templet in several pieces in

order to help facilitate the tool operations.

The use of this templet will not only be a help to getting all curved surfaces the

same, but will  also check  up on the various  new diameters on the model.  The

cylinder should never be in motion while the templet is being used.

CHAPTER VIII

FINISHING AND POLISHING

To get a high and lasting polish on wood, the work must be first sanded so as to be

perfectly smooth. In addition to this, open grained wood, such as oak, must be

properly filled with a wood filler. If properly sharpened tools have been used very

little sanding is required, and then worn sandpaper should be used as it does not cut

into the work as new paper cuts. Remember sandpaper is not to be used as a tool in

cutting down stock when working to dimensions. In using old sandpaper run the

lathe at a moderate speed to avoid burning the wood, especially on square or round

fillets. Keep the edges of the work sharp and do not wear them round. In using new

sandpaper use a fine grit (00 or 0) and move the paper from one end of the work to

the other slowly, so that no scratches result on the surface of the work.

The  work  may  be  finished  by  one  of  two  methods.  In  the  first  method  as  in

finishing ordinary cabinet work, the pieces should be stained and filled. In applying

filler, run the lathe at the slowest speed after the material has dried sufficiently to

rub into the pores of the wood. If the highlights are to be brought out, as in the case

of oak, stain and then give a light coat of shellac, and apply the filler after the

shellac is dry. The shellac keeps the dark filler from staining the flakes of the oak

darker, and the pores of the wood fill in as before. The pores become darker than

the flakes, and at the same time a smooth surface is produced. After the filler has

hardened the wood may be waxed or varnished.

The second method, or French polishing, is rather difficult to apply and requires a

little skill. A close grained wood, like maple, will be found more satisfactory for

the beginner. An open grained wood may be filled in the ordinary way, or the grain

may be filled by rubbing into the pores of the wood a combination of shellac, rotten

stone or pumice, oil and alcohol. Rotten stone is used for dark wood and pumice is

used for light wood. The wood may be left in the natural or stained as in the first

method. The mixture of shellac, rotten stone, oil and alcohol, is applied to the work

with a pad made of cotton waste, wrapped in cheese cloth to keep it from sticking

to the work. It should be about 1½" in diameter and ½" thick. Hold the pad over the

mouth of a bottle of shellac and tip the bottle so that the shellac comes in contact

with the pad. The shellac will remain clean in a bottle and will be handy. The

mouth of the shellac bottle should be about 1" in diameter and should be dipped

once. Do likewise with a bottle, having a mouth ½" in diameter, containing alcohol.

This should be dipped twice allowing the alcohol to dilute the shellac. Then drop

on a couple of drops of oil and rub over the pad evenly; this aids in distributing the

shellac properly and keeps the pad from sticking to the work. A bottle may also be

used for this. For the rotten stone use a pepper shaker so that it may be sifted on the

work as needed.

When the mixture has been applied to the pad, hold the pad against the work lightly

at first, until most of the moisture has been worked out of it, and then gradually

increase the pressure until the pad is almost dry. In putting on the first coat, use

more shellac and alcohol and just enough oil at all times to prevent the pad from

sticking to the work. However, the pad should not contain as much shellac that it

can be squeezed out with the fingers. When  the pad is dry, another mixture is

applied, and where open grained wood is used, rotten stone, or pumice stone, is

sprinkled on the work to  gradually  fill up  the pores and to build up a  smooth

surface. Run the lathe at a low speed, depending on the size of the piece that is

being polished. Allow the first coat to dry before applying a second coat for, if too

much is put on at any one time, the heat generated in the rubbing will cause the

shellac to pull, and it will form rings by piling up. These rings may be worked out

in two ways, either by a slight pressure of the pad on the rings or by cutting them

with alcohol applied to the pad. If too much alcohol is used it will cut through the

shellac and remove what has already been rubbed on. If at any time too much

shellac is used it will pile up and form rings. Too much rotten stone will cut down

the polish and by absorbing the mixture will leave the pad dry. If too much oil is

used the polish will become dull after a day or two.

After the first coat has hardened apply the second, but use less shellac and more

alcohol and just enough oil to prevent the pad from sticking. This may be done by

dipping the tip of a  finger in the oil and spreading it over the pad. The entire

mixture should be so that only a dampness can be felt on the pad. As the process

goes on less oil and shellac are used. All oil must be removed when applying the

last coat, or the piece will lose its polish. All the pores should be filled, and no

rings should be on the finished work. Where a natural finish is desired, apply a coat

of boiled linseed oil twelve hours before the work is to be polished. This will bring

out the grain and will also aid in applying the first coat; no oil need then be used in

the first coat.

A great amount  of practice  and patience is required  to  get  a first class polish.

Polishing can only be learned  by experience. Correct your troubles in properly

proportioning the mixture. Never use too much shellac as it will build up too fast

and will not harden, thus causing rings; or it will pull and catch to the pad, thus

forming bunches. The purpose of alcohol is mainly to dilute the shellac and to

prevent against putting it on the work too fast, but care must be taken not to use too

much alcohol to cut  the shellac entirely. The oil helps to distribute the shellac

evenly, but it must be removed when finishing the last coat, or the polish will not

remain. It also helps to keep the pad from sticking to the work.

It is impossible to obtain a polish that will be as lasting and rich by any method

other than the one described. For success it is essential to learn the proportions of

the mixture and to acquire skill in applying the materials by using exactly the right

pressure and the right movement of the pad.

CHAPTER IX

FACE-PLATE AND CHUCK TURNING

Face-plate and chuck turning open an entirely new field of work from that taken up

in  previous  chapters  of  this  book.  If  handled  correctly,  it  has  much  greater

educational and practical value than cylinder turning. From the practical standpoint

the field of work is broader and the models to be made are of much greater value.

Aside from this, trade methods and practices can be applied and a broad insight into

commercial work can be given the student.

In  some  details  of  chuck  turning  the  tool  operations  already  learned  can  be

employed, but for the most part they are entirely different. In order to preserve the

educational value of the work as brought out by skill and dexterity in handling

tools, it will be necessary to use the cutting method wherever possible. In some

instances that method will be impossible, and the scraping method must be used.

METHODS OF FASTENING STOCK

All the work thus far has been on models where the stock worked upon is held

between the live and dead centers. In face-plate and chuck turning the work is done

at the head stock only and the piece is supported by means of a face-plate, or

chuck, that is fastened to a face-plate, which is screwed onto the end of the live

spindle. There are three methods of fastening stock to the face-plate, and it depends

upon the nature of the exercise or model to be made which method is used.

1. SMALL SINGLE SCREW FACE-PLATE. For all work that does not require

deep cutting in the center, such as in towel rings, picture frames, etc., the small

face-plate with a single screw should be used.

Note:--Should  it  be  found  difficult  to  keep  the  block  from  working  loose  and

turning, it is a good plan to fold a piece of sandpaper, grit side out, and place it

between the face-plate and the stock.

2. LARGE SURFACE SCREW FACE-PLATE. For all work that does not require

deep cutting on the outside, such as exercises, jewel boxes, etc., as well as all large

stock, and all stock from which chucks are to be made, the large face-plate with the

surface screws should be used.

3. GLUING TO WASTE STOCK. A block of scrap wood is fastened to a face-

plate the same as for a chuck and surfaced off square. The block from which the

model is to be made is planed square on one side and glued to the block on the

face-plate with a sheet of paper between the two. To separate the model from the

chuck, after it is completed, place a chisel on the waste stock, 1/16" back of the

glue joint at such a point as will bring the chisel parallel to the grain of the model,

and strike lightly with a mallet. This will cause the paper to separate and the model

to become free.

This method will be found very convenient epecially on models where the base is

to be left straight. It will also be found to save much stock when working with

expensive woods.

LATHE ADJUSTMENTS

To get the best results in face-plate or chuck turning there should be no end play in

the spindle of the lathe. The spindle should always be tested out, and if any play is

found, should be adjusted before attempting any work. It is almost impossible to

make a true cut when such a condition obtains.

POSITION OF TOOL REST

For all face-plate and chuck turning the tool rest should be kept as close to the

stock as possible, the same as in spindle turning, regardless of the angle it may be

set. Vertically, the rest in most cases should be sufficiently below the center of the

stock to bring the center or cutting point of the tools used, when held parallel to the

bed  of  the  lathe,  even  with  the  center  of  the  stock.  This  last  condition  will

necessitate adjusting the height occasionally when changing from large to small

tools.

CHAPTER X

TOOL PROCESSES IN FACE-PLATE AND CHUCK TURNING

B-I--1-a

. Straight Cuts

1.  ROUGHING  OFF  CORNERS.  (¾"  GOUGE.)  FIG.  14.  The  tool  rest  is  set

crosswise to the bed of the lathe and parallel to the face of the stock.

Place the gouge on the rest with the handle well down. Roll the gouge to the left

until the grind which forms the cutting edge is perpendicular to the stock. The point

of contact should be slightly below the center or nose of the tool.

The handle of the gouge is then swung well to the back of the lathe or to the

operator's right. The gouge is then pushed forward into the stock and to the left,

making a shearing cut. The cut should not be too heavy. The starting point for this

cut should be a line which will indicate the largest diameter or circle that can be

made from the block.--This cut should be repeated until the corners are removed

from the block.

To complete the cutting of thick stock it will be found necessary to change the tool

rest to an angle of 45° with the bed of the lathe.

Fig. 14.

When hardwood is being turned it is sometimes advisable to saw the block almost

round with a compass saw or bandsaw, if one is to be had. Should this be done the

preceding steps are omitted.

The tool rest is then placed parallel with the lathe bed and a roughing cut is taken

with the gouge the entire thickness of the block.

The lathe should be run on second or third speed until the corners are removed, and

then changed to first speed.

2. CALIPERING FOR DIAMETER. The true diameter is then calipered the same

as in spindle work.

3. SMOOTHING CUT. A smoothing cut is taken with a skew chisel the same as in

spindle work.

Fig. 15.

4. ROUGHING CUT ON THE FACE. (¾" GOUGE.) FIG. 15. The rest is now

placed parallel to the bed of the lathe and slightly above the center of the spindle.

Place the gouge on the rest on its edge with the grind toward the stock and parallel

to the face to be surfaced. The nose of the gouge is the cutting point.

The handle is then raised and the cutting point is forced toward the center. A very

thin shaving should be taken. If the gouge is allowed to roll back so the grind above

the cutting point comes in contact with the wood it is sure to catch and gash the

wood.

5. SMOOTHING THE FACE. (SMALL SKEW CHISEL.) FIG. 16. For all work

up to 3" in diameter, the surface may be smoothed by using a small skew chisel in

the  same manner  as  in  squaring  the  ends of Stock  in  cylinder work.  (Step  6--

Exercise

A-I--1-a

, Straight Cuts.)

For larger work, place the chisel flat on the rest with the toe next to the stock and

the back edge of the chisel parallel to the face to be surfaced.

The point of the chisel is then forced toward the center of the stock, using the

straight back of the tool as a guide against the finished surface. Only a very thin cut

should be taken at a time.

Fig. 16.

Note:--While this operation may be termed a scraping cut, it will be found to be

much easier on the tool than if the cutting edge were held flat against the work as in

other scraping cuts.

The surface of the work should be tested for squareness by holding the edge of the

chisel or a straight edge across the face.

LAYING OFF MEASUREMENTS

In laying off measurements on the face of the stock a pencil compass or dividers

should be used. Set the compass or dividers to one-half the diameter of the circle

wanted. While one point is held at the exact center of the stock, which is easily

located  while  the  stock  is  revolving,  the  other  is  brought  in  contact  with  the

revolving stock until a circle of the correct diameter is marked.

Fig. 17.

Should the center of the stock be cut away, rendering this method impossible, the

following method may be used: Set the compass or dividers to the exact diameter

wanted. Place one point in contact with the stock a little to one side of the required

line on the part that is to be cut-away. Bring the other point to the stock and see if it

touches the line first made. If not, move the first point until the two points track in

the same line.

Fig. 18.

The rest should be set at the exact center for measuring.

All measurements on the edge of the stock can be made with pencil and rule as in

cylinder turning.

B-I--2-a

. Shoulder Cuts

1.  EXTERNAL  SHOULDERS.  FIG.  18.  The  surplus  stock  at  each  successive

shoulder  is  roughed  out  with  a  ¾"  gouge,  keeping  well  outside  the  finished

measurements. The gouge for this work is held in the same position as described in

B-I--1-a

, Step 1, for Roughing Off Corners.

Fig. 19.

2. For the finishing cut a small skew chisel is used, and the process is the same as

that used in squaring ends of stock. Both the vertical and horizontal shoulders can

be handled easily by this method. Fig, 19.

3. INTERNAL SHOULDERS. For internal shoulder cutting the same methods may

be used for roughing out and cutting the horizontal shoulders, but for the vertical or

base  shoulder  it  will  be  necessary  to  use  the  scraping  process.  (See  "Use  of

Scraping Tools.")

B-I--3-a

. Taper Cuts

Taper cutting will not be found hard as the gouge and skew chisel are used in the

same manner as described in

B-I--1-a

, Steps 4 and 5. After the stock has been

roughed away with the gouge to the approximate angle desired, a smoothing cut is

taken with the skew. Care should be taken that the skew chisel is held at the exact

angle of the taper desired.

B-I--4-a

. V Cuts

V cutting will also be found easy as the tool process is exactly the same as that used

in spindle turning. Exercise

A-I--4-a

. Fig. 20.

B-I--5-a

. Concave Cuts

Place the ¾" gouge on the rest with the handle parallel to the bed of the lathe. Roll

the gouge on its edge and swing the handle so that the grind is perpendicular to the

stock with the nose of the tool as the cutting point.

Fig. 20.

Force the gouge forward into the wood. As soon as the cut is started, the handle is

lowered and swung to the left; (if cutting the left side of the concave) at the same

time the tool is rolled back toward its original position. This movement brings the

cutting point farther down on the lip and the grind, resting on the side of the cut,

will force the gouge sidewise and will form one-quarter of the circle. Fig. 21.

Fig. 21.

This cut is continued from alternate side until the concave is nearly to size. The cut

should be tested with a templet before the finishing cut is taken.

B-I--6-a

. Convex Cuts

Rough out the stock between the beads with a parting tool.

Hold the edge of the gouge on the rest with the handle, parallel to the bed of the

lathe, to make the nose the cutting point.

Swing the handle to the left so that the grind will form a tangent to the bead at its

highest point.

The gouge is then forced into the stock and to the right; at the same time the handle

is swung to the right; keeping the grind tangent to the bead at the point of contact.

Fig. 22. This cut is continued until the base of the bead is reached.

B-I--7-a

. Combination Cuts

As  in spindle turning,  a combination  exercise  should be  given  at  this  point to

provide an opportunity for studying out the best methods of working the various

cuts just described into a finished product.

USE OF SCRAPING TOOLS

When scraping is to be employed, it should be done with only those tools that are

made for that purpose, i.e., Square Nose, Round Nose, Spear Point, Right and Left

Skew. The handling of these tools will be found easy. The only point to remember

is that they should be held flat on the tool rest and parallel to the bed of the lathe

when in use.

In general practice the ordinary skew chisel should not be used as a scraping tool,

for the cutting edge is not sharpened to withstand the heavy strain required by such

work. Should it be necessary, however, to use a skew chisel as a scraper, the tool

should be held so that the top grind is parallel to the bed of the lathe while in use.

INTERNAL BORING

In roughing out the center for Napkin Rings, Jewel Boxes, etc., the quickest method

is to work it out with a small gouge.

Place the gouge on the rest parallel to the bed of the lathe, having the point even

with the center of the stock.

Force the gouge into the wood until a hole is bored to the depth required. If the hole

is deeper than 1", remove the tool often and clear out the shavings in order not to

burn the point.

In order to enlarge the hole to the proper size the point of the gouge is pressed

against the left side of the hole a little above the center and a shearing cut is taken.

To obviate the danger of the tool catching, all cuts should start from the back of the

hole and proceed toward the front.

Fig. 22.

B-III--8-a

. Sphere

After the sphere is turned as nearly perfect as is possible when working between

centers (Steps 1 to 4) it is cut free from the waste stock and is centered in a chuck.

The chuck is made of any soft wood and should be cut in the end grain, which will

insure equal pressure on all sides. Equal pressure cannot be obtained if the Chuck is

cut in cross grain wood, owing to the tendency of side grain to give more than the

end  grain.  The  sphere  should  be  forced  into  the  chuck  with  slightly  over  half

protruding. Very thin cuts should be taken and the sphere should be revolved one-

quarter turn after each until true. As the sphere becomes smaller during the cutting,

it will be necessary to cut the face of the chuck down and bore the hole deeper and

smaller in order to keep more than half of it protruding at all times.

Mirror (See Pages

299

-

301

).

To remove the sphere tap the chuck lightly with a hammer just above it, at the same

time pull out on the sphere.

CHAPTER XI

SPIRAL TURNING

Spiral turning is a subject that has received very little attention by most schools in

which wood turning is taught. Spiral work is seen in antique furniture and also in

the modern furniture of the present day. It seems that it takes the wheel of fashion

about a century to make a complete turn, for what our forefathers neglected and

destroyed the people of the present day value and cherish.

Spiral work gives excellent practice in shaping and modelling wood. It brings into

play the principle of the helix as used in cutting threads, etc.; and its form, size and

shape may be varied according to the taste of the individual. As in threads so in

spiral  work  we  have  single  and  double  spirals,  and  their  form  and  proportion

depend upon their use and application in furniture making. A variation of the spiral

may be made in several ways: First, by changing the number of turns of the spiral

on  a  straight  shaft;  second,  by  running  a  spiral  on  a  tapered  shaft;  third,  by

changing the shape or form of the spiral itself; and fourth, by making more than

one spiral on a shaft. It is uncommon to see ten or twelve spirals running around a

single shaft.

Some of the forms of the above types are fully taken up and explained in the work

that is to follow.

PLATES

B-V--1-a

,

B-V--1-a’

. SINGLE SPIRAL. STRAIGHT SHAFT

To work out a single spiral for a pedestal proceed as follows:

1. Turn a cylinder 2¼" in diameter. Make the ends slightly larger in order that the

design may be turned on each, after the spiral has been worked out.

2. Lay off spaces 2-1/16" apart on the cylinder while the spindle is turning in the

lathe and divide each of these into four equal parts. Each one of these large spaces

represents  one  turn  of  the  spiral.  A  good  proportion  is  slightly  less  than  the

diameter of the cylinder; thus the diameter of the cylinder equals 2¼" and the width

of the space 2-1/16".

3. On the cylinder parallel to the axis draw lines A-A B-B C-C D-D. These lines

should be 90° apart as shown in the top diagram (Plate

B-V--1-a’

). Line D-D is on

the other side of the cylinder as shown in the top and middle diagrams.

Fig. 23.

4. Start on line A-A at point X, circle 1, and draw a line connecting it with line B-B

on circle 1’. Then connect B-B on circle 1’ with C-C on circle 2 and so on until a

spiral has been drawn the entire length of the cylinder. This line will form the ridge

of the spiral as shown in the middle diagram.

5. Next begin on line C-C at circle 1, and draw a line connecting it with D-D on

circle 1’ then to line A-A on circle 2, and so on as before. This spiral represents the

center of the groove or the portion which is to be cut away. This is not shown in the

diagram because more or less confusion would be caused with the line representing

the ridge of the spiral.

6. Begin on line C-C at circle 1, and saw to a depth of ¾". Saw the entire length of

the cylinder leaving about 1½" at the ends. Do not follow the line here, but switch

off gradually and follow circles 1 and 15, so as to allow the spiral to begin and end

gradually and not abruptly.

7. Rough out with a knife or chisel by cutting on both sides of the saw cut. Then

use a wood rasp to finish shaping out the spiral. When properly shaped out allow

the lathe to turn slowly and smooth with sandpaper by following the spiral as the

lathe turns.

Fig. 24.

Fig. 24-a.

Fig. 25.

8. Cut the design on both ends of the cylinder and polish.

PLATES

B-V--2-a

,

B-V--2-a’

,

B-V--2-a’’

;. SINGLE SPIRAL. TAPERED SHAFT

To lay off a single spiral for the electric lamp shown in Figs. 24 and 24a proceed as

follows:

1. Select your wood and bore a hole through it. Plug the hole and center the piece in

the lathe. This insures getting the hole exactly in the center, and it will not be cut

into while the cutting of the groove of the spiral proceeds. A groove may also be

cut in two pieces of stock and glued together to form a hole through the stock.

2. Turn a cylinder 2½" in diameter, tapering it to 1½" at the one end; this part

should be 12?" long. Both ends should be left larger than 2½" as the lower and

upper designs must be cut here.

3. Let the spindle revolve in the lathe and draw circles as shown in the layout (Plate

B-V--2-a’

). The number of circles will vary with the taper. Since seven turns are

needed in the present spiral, 28 circles will be necessary--four circles for each turn

of the spiral as shown in the middle diagram. A good proportion to follow is to

measure the diameter of the spindle at circle 2 and lay off this distance from circle

1 to circle 3. Then measure the diameter at circle 4 and lay off this diameter from

circle 3 to circle 5 and so on until all circles have been made. Then divide these

large divisions into four equal parts.

4. Draw four lines the entire length of the spindle, each 90° apart as shown by the

heavy lines in the middle diagram. The heavy circles of the same diagram represent

the complete turns of the spiral.

5. Lay out the line representing the ridge of the spiral as shown in the middle

diagram. Begin on circle 1, where the straight line crosses it, draw to circle 1’ at the

point where the next straight line crosses it, then to 2--2’--3--3’ and so on until the

end is reached. This forms the ridge of the spiral as shown in diagram 3. Next it

may be more convenient to draw another line representing the groove. In this case

begin at point X in the middle diagram, opposite the point where first started, and

continue in the preceding manner, making this line parallel to the other line.

6. Saw on the line last made, being careful not to saw too deeply. The depth must

be ¼" less than half the diameter of the spindle where the cut is made. This saw cut

forms the groove of the spiral. The groove is then cut out by hand with a chisel or

knife, by working down the wood on both sides of the saw cut. After the spirals

have been roughed out, a rasp is used to finish shaping them. The work is then

sandpapered smooth, while the spindle is revolved slowly in the lathe.

7. Cut designs on the ends of the cylinder and polish.

PLATES

B-V--2-b

,

B-V--2-b’

. DOUBLE SPIRAL. TAPERED SHAFT

To work out a double spiral for the electric lamp illustrated in Fig. 25 proceed as

follows:

1. Turn up the spindle in the usual manner. Since the base of the shaft is larger than

the top, the spiral must also be in proportion and lines A-A’, B-B’, C-C’, D-D’, and

E-E’, are drawn around the shaft. To get the approximate spacing from circles A-A

to B-B measure the diameter at A-A’ plus about 3/16" and lay off from A-A’ to B-

B’. Then take the diameter of B-B’ plus about 3/16" and lay off from A-A’ to B-B’.

Then take the diameter at B-B’ plus about 3/16" and lay off from circle B-B’ to C-

C’ and so on. If the shaft is tapered more, a different proportion must be used. Also

if it is desired to have the twist wind around the shaft three times, a variation must

be made in the number of circles.

Fig. 27.

2. If it is desired to have the twist wind around the shaft twice, draw circles 1-1’, 2-

2’, 3-3’, and 4-4’ and the spaces will grow proportionately smaller at the small end.

3. Draw four lines running lengthwise on the spindle and 90° apart as shown in the

midde figure in heavy lines (Plate

B-V--2-b’

).

4. Begin at A and draw a curved line to where the 90° line crosses circle 1-1’. From

there extend the line to where the next 90° line crosses circle B-B’ at point B’.

Continue in this manner until the other end of the shaft is reached. Begin at A’ and

draw a line on the opposite side of the shaft. These two lines running around and

along the shaft form the grooves while the portion in between forms the beads of

the double spiral.

5. Saw to the desired depth, being ¼" less than half the diameter at the point where

cut. With a chisel or knife form the grooves and beads. It is necessary to be careful

about not ending the grooves too abruptly. (See point 6 in Plates

B-V--1-a

,

B-V--1-

a’

.) Smooth with a rasp and sandpaper while the lathe is revolving slowly.

6. Cut the design on the ends and polish.

PLATES

B-V--3-a

,

B-V--3-a’

.

DOUBLE

GROOVE

SPIRAL.

STRAIGHT

SHAFT

To work out the double groove spiral for the magazine holder illustrated, proceed

as follows:

1. Square up the stock to 1?". Center carefully and turn the design on both ends as

shown, in the upper diagram (Plate

B-V--3-a’

). Turn the cylinder between the top

and bottom, making it 5½" long and 1?" in diameter.

Fig. 26.

2. Divide the cylinder into two equal parts. Each part represents one revolution of

the spiral.

3. Divide each half into four equal parts as shown in the top and center diagrams

(Plate

B-V--3-a’

),  1-1’,  2-2’,  3-3’  and  so  on.  The  proportion  of  the  distance

between these circles should be one-half the diameter of the cylinder.

4. Draw lines A-A, B-B, C-C, and D-D, parallel to the axis of the cylinder 90°

apart.

5. With a band 3/16" wide of any substantial material (preferably a narrow strip of

tin or a watch main spring) begin on the line A-A at circle 1, and connect circle 1’

at line B-B, and then connect circle 2 at C-C, and so on until the spiral is made the

entire length. Mark on both sides of the 3/16" band so as to keep the spiral parallel.

6. Next begin at the line C-C where circle 1 crosses it and connect from here to 1’

at B-B. Proceed as in Step 5, as shown in the center diagram.

7. Now erase the extreme ends of the spiral near circles 1 and 5, and deviate from

the original spiral and follow the circles in a more parallel direction so as to allow

the spiral to begin and end  gradually and  not too  abruptly.  Refer  to the  lower

diagram for this.

8.  Cut  out  portions  of  wood  between  the  bands  previously  marked  around,  as

shown in the lower figure. The wood should be cut out with a knife so as to leave

the corners sharp on the narrow bands. The portion cut out should be a semi-circle

and can be sanded by making a spindle a little smaller than the distance between

the bands and fastening sandpaper on the spindle. Place in the lathe and hold the

spiral  on  the  sandpaper  cylinder  at  an  angle  so  that  the  spiral  will  fit.  Turn

gradually and the sandpaper will smooth up the portion between the bands and true

it up. At the ends where the grooves are smaller, use a smaller stick around which

sandpaper has been wound and work out by hand.

9. It is well to cut straight down, about 1/32" deep, along the lines marking out the

narrow bands. Then the wood will not be so likely to split while removing the stock

which forms the grooves between the bands.

10. Cut out the mortises in the square portions which have been left at both ends.

Make the frame work for the sides and cane. Glue together and polish.

Note:--By making the posts smaller and using the same construction for a side a

nice looking book stall may be made. The proportions for the posts are the same as

mentioned in Step 3.

[Transcribers note: There are 142 line art illustrations after this point in the book.

See the

Classification of Plates

for all of them.]